Perfect 👍 Let’s build a sensor-to-front-end mapping for your crane predictive maintenance system.

🔧 Sensors & Front-End Mapping

1. Vibration Sensors (Bearings, Gearbox, Motors)

- Sensor type: IEPE piezoelectric accelerometer (e.g., PCB Piezotronics, Dytran, Brüel & Kjær).

- Front-end:

  - IEPE front-end with constant current excitation (2–20 mA at ~24 V).

  - AC coupling → high-pass filter (to remove bias).

  - Gain stage → scale to ADC input (±2.5 V or similar).

- ADC needs: High sampling (≥25 kHz per channel) for FFT-based condition monitoring.

2. Strain / Stress Monitoring (Boom, Hook, Joints)

- Sensor type: Strain gauge or load cell (Wheatstone bridge).

- Front-end:

  - Bridge front-end:

    - Precision excitation (2.5 V / 5 V).

    - Instrumentation amplifier (INA, e.g., TI INA333, AD8421).

    - Low-pass filter (to suppress noise).

- ADC needs: High-resolution (24-bit SAR/ΔΣ ADC, e.g., ADS124S08) for µV sensitivity.

3. Temperature Sensors (Motors, Bearings, Hydraulics)

- Sensor type:

  - RTD (Pt100, Pt1000) or

  - Thermocouples for higher ranges.

- Front-end:

  - RTD → Excitation current source + instrumentation amplifier.

  - Thermocouple → Cold junction compensation + differential amplifier.

- ADC needs: High-resolution (16–24 bit, low sample rate ~10 Hz).

4. Current Sensors (Motors, Drives)

- Sensor type:

  - Hall-effect sensors (e.g., ACS712).

  - Shunt resistors for precise low-level current.

- Front-end:

  - Hall-effect → Direct ADC input (may need scaling/filter).

  - Shunt → Differential amplifier (e.g., INA199).

- ADC needs: 12–16 bit, moderate sample rate (~kHz).

5. Position / Angle (Crane Boom, Hook)

- Sensor type:

  - Rotary encoders (digital → no front-end, goes to MCU).

  - Potentiometer (analog → needs buffer amplifier).

- Front-end:

  - Potentiometer → Voltage divider + buffer op-amp.

- ADC needs: 12–16 bit, slow sample rate (~10–50 Hz).

📊 Summary Table

| Sensor | Use in Crane | Front-End Needed | Example ICs |
| --- | --- | --- | --- |
| IEPE Accelerometer | Vibration on gearbox, bearings | IEPE front-end (CCS + AC coupling + amp) | TI OPAx, IEPE driver |
| Strain Gauge / Load Cell | Boom stress, hook load | Bridge front-end (excitation + INA) | INA333, AD8421 |
| RTD / Thermocouple | Motor, hydraulic temp | Excitation + amplifier + CJC | MAX31865 (RTD), MAX31855 (Thermocouple) |
| Current Sensor | Motor health monitoring | Shunt + diff amp / Hall sensor | INA199, ACS712 |
| Encoder / Potentiometer | Boom / hook position | Buffer amplifier / digital interface | TLV9001, MCU GPIO |

🚀 System Flow

Sensor → (IEPE/Bridge/Analog front-end) → TI Precision ADC → STM32 MCU → Local Analysis (FFT, anomaly detection, ML) → Predictive Maintenance Alerts

👉 Next step: I can help you choose specific TI ADCs and STM32 MCU family (F4, F7, H7, or industrial-grade STM32MP1) depending on the number of sensors, required sampling rate, and computational load.

Do you want me to design the system architecture with sensor count, front-end, ADC, and MCU selection for your crane monitoring setup?

Great question 👍 Let’s break it down — since you’re planning a predictive maintenance and health monitoring system for a crane, you’ll come across sensors that often need a front-end interface before the ADC (like your TI ADC).

1. IEPE Front-End

IEPE = Integrated Electronics Piezo-Electric

- Many industrial vibration/accelerometer sensors are IEPE type.

- These sensors have a built-in charge amplifier but require a constant current excitation (typically 2–20 mA at 18–30 V).

- The signal from the sensor is an AC-coupled voltage superimposed on the excitation bias.

- The IEPE front-end provides:

  - Constant current excitation to the sensor.

  - AC coupling and bias removal.

  - Signal conditioning (amplification, filtering) so the ADC can read the vibration data.

👉 Example use: Mounting an IEPE accelerometer on crane gearbox/motor housing to monitor vibration.

2. Bridge Front-End

- This is for sensors based on Wheatstone bridge configurations (common in strain gauges, load cells, torque sensors, pressure sensors).

- These sensors output millivolt-level differential signals.

- The bridge front-end provides:

  - Stable excitation voltage for the bridge.

  - Precision instrumentation amplifier (to amplify µV–mV signals to ADC range).

  - Filtering to remove noise before digitization.

👉 Example use: Strain gauge sensors on crane boom or hook to measure stress and load.

3. Why You Need Them

Without proper front-end circuits:

- IEPE accelerometers won’t even power up (they need current bias).

- Strain gauges produce signals too small for the ADC (and very noisy).

So, the front-end = sensor interface electronics that make raw sensor signals usable for your TI ADC and STM MCU.

✅ In your project (Crane Predictive Maintenance):

- IEPE front-end → for vibration sensors on gearbox, motors, bearings.

- Bridge front-end → for strain/load sensors on boom, joints, hook.

- Both feed into TI precision ADC → processed by STM32 MCU running predictive algorithms (FFT, anomaly detection, ML).

Would you like me to map exact sensor types (accelerometer, strain gauge, current, temperature, etc.) to the correct front-end (IEPE, bridge, or simple analog) for your crane system?