Absolutely 🚀
Let’s walk step-by-step through the flow of the driver + main code so you see how everything connects.

🔄 High-Level Flow

- Hardware setup (CubeMX/Board)

  - Configure SPI1 + SPI2 in mode 1 or 3 (per ADS131M08 datasheet: CPOL=1, CPHA=1).

  - Configure GPIO pins for each ADC:

    - CS (chip select, output)

    - RESET (output)

    - DRDY (input, signals new data available)

  - Configure UART (for debugging/printing).

- Create driver handles (ADS131_Handle_t)
Each ADC instance stores pointers to its SPI, CS, RESET, DRDY pins, and a flag for CRC enable.

- ADS131_Handle_t ads1 = { .hspi=&hspi1, .CS_Port=..., .crc_enable=1 };

- ADS131_Handle_t ads2 = { .hspi=&hspi2, .CS_Port=..., .crc_enable=0 };

- Initialize ADCs (ADS131M08_Init)
For each ADC:

  - Toggle RESET pin (hardware reset).

  - Send RESET command (software reset).

  - Send UNLOCK command (allow register writes).

  - Write configuration registers:

    - MODE → 24-bit word, OSR=1024, chop on.

    - CLOCK → default external clock.

    - GAIN1/GAIN2 → all channels gain=1.

    - CFG → internal reference on, CRC on/off depending on crc_enable.

  - Write CHn_CFG for each channel → enable.

  - Send LOCK command (optional, block config writes).

  - Send START command → continuous conversions.

🔑 After this step, the ADC is sampling continuously and presenting data frames on SPI whenever DRDY goes low.

- Main loop

  - The MCU polls DRDY pin (or could use interrupt).

  - When DRDY is low → call ADS131M08_ReadDataFrame().

    - Builds a SPI transfer (TX=0s, RX=frame).

    - Reads either:

      - 27 bytes (status + 8 channels × 3 bytes) if CRC disabled.

      - 29 bytes (same + 2 CRC bytes) if CRC enabled.

    - If CRC enabled → compute CRC16 on all but last 2 bytes, compare to received CRC. If mismatch → HAL_ERROR.

    - Parse each channel’s 24-bit signed sample and sign-extend to 32-bit int.

  - Store/print values via UART.

- Output
Example log:

- ADS1 init OK with CRC

- ADS2 init OK without CRC

- ADC1 CH0=123456 CH1=-234567

- ADC2 CH0=112233 CH1=-445566

📊 Code Flow Diagram (simplified)

main()

├─ HAL_Init()

├─ MX_SPI1/2_Init(), MX_GPIO_Init(), MX_USART1_UART_Init()

├─ ADS131M08_Init(&ads1)   --> config ADC1, enable CRC

├─ ADS131M08_Init(&ads2)   --> config ADC2, no CRC

└─ while(1)

├─ if DRDY1 == 0 → ADS131M08_ReadDataFrame(&ads1, adc1_samples)

│                     └─ check CRC (if enabled), parse 8 channels

│                     └─ return samples[]

├─ if DRDY2 == 0 → ADS131M08_ReadDataFrame(&ads2, adc2_samples)

└─ print values over UART

✅ Key Points

- SPI1 and SPI2 run independently → can capture from 2 ADC boards simultaneously.

- Polling DRDY → ensures data is fresh (no missed samples if loop fast enough).

- CRC enable flag → optional per device; automatically verified in ReadDataFrame.

- Samples[] array → always holds latest signed 32-bit values (one per channel).

👉 Do you want me to also draw a timing diagram of SPI transfers (RESET → INIT → continuous data read) so you can visualize how data moves between STM32 and ADS131M08?

ReadDataFrame with CRC check

/* Compute CRC16 (polynomial 0x8005, init=0xFFFF, MSB-first) */

static uint16_t ads131_crc16(const uint8_t *data, uint16_t len)

{

uint16_t crc = 0xFFFF;

for (uint16_t i = 0; i < len; i++) {

crc ^= ((uint16_t)data[i] << 8);

for (uint8_t b = 0; b < 8; b++) {

if (crc & 0x8000)

crc = (crc << 1) ^ 0x8005;

else

crc <<= 1;

}

}

return crc;

}

/* Read one data frame:

* - Normal: 27 bytes (3 status + 8×3 channel)

* - With CRC: 29 bytes (extra 2 at end)

*/

HAL_StatusTypeDef ADS131M08_ReadDataFrame(ADS131_Handle_t *h, int32_t samples[ADS131M08_NUM_CHANNELS])

{

if (!h || !samples) return HAL_ERROR;

/* Max frame length = 29 bytes */

uint8_t tx[32] = {0};

uint8_t rx[32] = {0};

/* Always read full frame with possible CRC */

uint16_t frame_len = ADS131M08_FRAME_BYTES;

/* If CRC enabled → add 2 bytes */

uint16_t cfg_val;

if (ADS131M08_ReadRegister(h, ADS131M08_REG_CFG, &cfg_val) == HAL_OK) {

if (cfg_val & ADS131M08_CFG_CRCEN_MASK) {

frame_len += 2;

}

}

ADS131_CS_LOW(h);

if (HAL_SPI_TransmitReceive(h->hspi, tx, rx, frame_len, HAL_MAX_DELAY) != HAL_OK) {

ADS131_CS_HIGH(h);

return HAL_ERROR;

}

ADS131_CS_HIGH(h);

/* CRC check if present */

if (frame_len > ADS131M08_FRAME_BYTES) {

uint16_t received_crc = ((uint16_t)rx[frame_len - 2] << 8) | rx[frame_len - 1];

uint16_t calc_crc = ads131_crc16(rx, frame_len - 2);

if (received_crc != calc_crc) {

return HAL_ERROR; /* CRC mismatch */

}

}

/* Parse channel data (skip 3 status bytes) */

for (int ch = 0; ch < (int)ADS131M08_NUM_CHANNELS; ++ch) {

uint32_t idx = 3u + (uint32_t)ch * 3u;

int32_t raw24 = ((int32_t)rx[idx] << 16) |

((int32_t)rx[idx + 1] << 8) |

(int32_t)rx[idx + 2];

/* sign-extend 24 → 32 */

if (raw24 & 0x00800000) raw24 |= (int32_t)0xFF000000;

samples[ch] = raw24;

}

return HAL_OK;

}

✅ Behavior now:

- Checks CFG.CRCEN before deciding frame length.

- Reads 27 bytes if CRC disabled, 29 bytes if CRC enabled.

- Computes CRC16 over all bytes except the last 2.

- Returns HAL_ERROR if CRC mismatch.

- Otherwise returns signed 24-bit channel samples in samples[].

ads131m08.c

/* ads131m08.c

* Multi-instance driver implementation for ADS131M08 using STM32 HAL

*/

#include "ads131m08.h"

#include <string.h>

/* Helper: transmit/receive bytes over SPI for given ADS handle.

* Blocking call with HAL_MAX_DELAY (simple polling approach).

*/

static HAL_StatusTypeDef spi_txrx(ADS131_Handle_t *h, uint8_t *tx, uint8_t *rx, uint16_t len)

{

return HAL_SPI_TransmitReceive(h->hspi, tx, rx, len, HAL_MAX_DELAY);

}

/* Send a 16-bit command (MSB first) */

HAL_StatusTypeDef ADS131M08_SendCommand(ADS131_Handle_t *h, uint16_t cmd)

{

uint8_t tx[2] = { (uint8_t)((cmd >> 8) & 0xFF), (uint8_t)(cmd & 0xFF) };

uint8_t rx[2];

ADS131_CS_LOW(h);

if (spi_txrx(h, tx, rx, 2) != HAL_OK) {

ADS131_CS_HIGH(h);

return HAL_ERROR;

}

ADS131_CS_HIGH(h);

/* small settling delay recommended */

HAL_Delay(1);

return HAL_OK;

}

/* Write 1 x 16-bit register (address, then 2 bytes value) using WREG op (count = 0) */

HAL_StatusTypeDef ADS131M08_WriteRegister(ADS131_Handle_t *h, uint8_t addr, uint16_t value)

{

uint8_t tx[4];

uint8_t rx[4];

tx[0] = (uint8_t)(ADS131M08_WREG_OPCODE | (addr & 0x3Fu));

tx[1] = 0x00u; /* count - 1 = 0 */

tx[2] = (uint8_t)((value >> 8) & 0xFF);

tx[3] = (uint8_t)(value & 0xFF);

ADS131_CS_LOW(h);

if (spi_txrx(h, tx, rx, 4) != HAL_OK) {

ADS131_CS_HIGH(h);

return HAL_ERROR;

}

ADS131_CS_HIGH(h);

HAL_Delay(1);

return HAL_OK;

}

/* Read 1 x 16-bit register using RREG op (count = 0) */

HAL_StatusTypeDef ADS131M08_ReadRegister(ADS131_Handle_t *h, uint8_t addr, uint16_t *value)

{

if (!value) return HAL_ERROR;

uint8_t tx[2];

uint8_t rx[2];

tx[0] = (uint8_t)(ADS131M08_RREG_OPCODE | (addr & 0x3Fu));

tx[1] = 0x00u; /* count-1 = 0 */

ADS131_CS_LOW(h);

if (HAL_SPI_Transmit(h->hspi, tx, 2, HAL_MAX_DELAY) != HAL_OK) {

ADS131_CS_HIGH(h);

return HAL_ERROR;

}

/* read 2 bytes reg value */

if (HAL_SPI_Receive(h->hspi, rx, 2, HAL_MAX_DELAY) != HAL_OK) {

ADS131_CS_HIGH(h);

return HAL_ERROR;

}

ADS131_CS_HIGH(h);

*value = ((uint16_t)rx[0] << 8) | rx[1];

HAL_Delay(1);

return HAL_OK;

}

/* Initialize ADS131M08: hardware reset, software RESET, UNLOCK, write some registers, LOCK, START.

* NOTE: register values here are example placeholders — change to your required config.

*/

HAL_StatusTypeDef ADS131M08_Init(ADS131_Handle_t *h)

{

if (!h) return HAL_ERROR;

ADS131_CS_HIGH(h);

/* hardware reset pulse */

ADS131_RESET_LOW(h);

HAL_Delay(10);

ADS131_RESET_HIGH(h);

HAL_Delay(10);

/* software reset */

if (ADS131M08_SendCommand(h, ADS131M08_CMD_RESET) != HAL_OK) return HAL_ERROR;

HAL_Delay(5);

/* unlock registers */

if (ADS131M08_SendCommand(h, ADS131M08_CMD_UNLOCK) != HAL_OK) return HAL_ERROR;

HAL_Delay(2);

/* Example: MODE = 0x0000 (normal), CLOCK default, GAIN regs placeholder

* Replace these with desired register configurations from datasheet.

*/

(void)ADS131M08_WriteRegister(h, 0x02u, 0x0000u); /* MODE */

(void)ADS131M08_WriteRegister(h, 0x03u, 0x0000u); /* CLOCK */

(void)ADS131M08_WriteRegister(h, 0x04u, 0x1111u); /* GAIN1 (example) */

(void)ADS131M08_WriteRegister(h, 0x05u, 0x0000u); /* GAIN2 (example) */

(void)ADS131M08_WriteRegister(h, 0x06u, 0x0000u); /* CFG: internal ref */

/* Enable channel configs — here example enabling each channel with minimal config.

* Ch N cfg register address = 0x09 + N*5 (each channel has 5 registers)

*/

for (uint8_t ch = 0; ch < ADS131M08_NUM_CHANNELS; ++ch) {

uint8_t ch_addr = (uint8_t)(0x09u + (uint8_t)(5u * ch));

(void)ADS131M08_WriteRegister(h, ch_addr, 0x0001u); /* enable channel (example) */

}

/* lock registers (optional) */

(void)ADS131M08_SendCommand(h, ADS131M08_CMD_LOCK);

HAL_Delay(2);

/* start conversions */

if (ADS131M08_SendCommand(h, ADS131M08_CMD_START) != HAL_OK) return HAL_ERROR;

HAL_Delay(2);

return HAL_OK;

}

/* Read one data frame (blocking): returns ADS131M08_NUM_CHANNELS signed 24-bit samples sign-extended into int32_t.

* Frame size: 3 status bytes + 8 * 3 channel bytes = 27 bytes.

*/

HAL_StatusTypeDef ADS131M08_ReadDataFrame(ADS131_Handle_t *h, int32_t samples[ADS131M08_NUM_CHANNELS])

{

if (!h || !samples) return HAL_ERROR;

uint8_t tx[ADS131M08_FRAME_BYTES];

uint8_t rx[ADS131M08_FRAME_BYTES];

memset(tx, 0x00, sizeof(tx));

memset(rx, 0x00, sizeof(rx));

ADS131_CS_LOW(h);

if (spi_txrx(h, tx, rx, (uint16_t)sizeof(tx)) != HAL_OK) {

ADS131_CS_HIGH(h);

return HAL_ERROR;

}

ADS131_CS_HIGH(h);

/* parse channel data (skip first 3 status bytes) */

for (int ch = 0; ch < (int)ADS131M08_NUM_CHANNELS; ++ch) {

uint32_t idx = 3u + (uint32_t)ch * 3u;

int32_t raw24 = ((int32_t)rx[idx] << 16) | ((int32_t)rx[idx + 1] << 8) | (int32_t)rx[idx + 2];

/* sign extend 24->32 */

if (raw24 & 0x00800000) raw24 |= (int32_t)0xFF000000;

samples[ch] = raw24;

}

return HAL_OK;

}